UNIVERSITY OF EASTERN AFRICA, BARATON

COURSE OF STUDY: NETWORKING AND COMMUNICATION SYSTEMS

COURSE: COMPUTER NETWORK DESIGN

NAME & SCHOOL ID: ODERO KEN MYRON - SODEMY2211
AfriTech Solutions Ltd.

Enterprise WAN Network Design Documentation

Complete Network Design (CND) Project

| Project Name: | AfriTech Solutions Network Infrastructure |
| --- | --- |

| Organization: | AfriTech Solutions Ltd. |
| --- | --- |
| Project Type: | Complete Network Design (CND) |
| Network Type: | Multi-Site Wide Area Network |
| Sites: | 2 Geographic Locations (Nairobi HQ, Mombasa Branch) |
| Total Devices: | 362 (13 infrastructure + 320 end devices + 8 APs) |
| Date: | April 2026 |
| Status: | Complete - Ready for Implementation |

# Table of Contents

1. Executive Summary

1.1 Project Overview

1.2 Business Requirements

1.3 Solution Summary

2. Network Architecture

2.1 Network Topology

2.2 Site Breakdown

2.3 Physical Infrastructure

3. IP Addressing Scheme

3.1 Site 1 - Nairobi HQ (Class B)

3.2 Site 2 - Mombasa Branch (Class C)

3.3 WAN Links and Router Interfaces

4. VLAN Design

4.1 VLAN Segmentation Strategy

4.2 VLAN Distribution

5. Routing & Switching Protocols

5.1 OSPF Configuration

5.2 HSRP Gateway Redundancy

5.3 Spanning Tree Protocol

6. Network Services

6.1 DHCP Configuration

6.2 DNS Services

6.3 NAT/PAT

7. Wireless Infrastructure

7.1 Access Point Deployment

7.2 Wireless Security

8. Security Implementation

8.1 Access Control

8.2 Port Security

8.3 DHCP Snooping & DAI

9. Equipment Specifications

9.1 Hardware Inventory

9.2 End Device Distribution

10. Testing & Verification

10.1 Testing Methodology

10.2 Verification Results

11. Implementation Plan

11.1 Deployment Phases

11.2 Risk Mitigation

12. Conclusion & Recommendations

12.1 Project Summary

12.2 Future Enhancements

# 1. Executive Summary

## 1.1 Project Overview

AfriTech Solutions Ltd. has designed and implemented a comprehensive enterprise-grade Wide Area Network (WAN) connecting two geographically separated sites: the Nairobi Headquarters and the Mombasa Branch office. This multi-site network architecture supports 320 end devices across 8 departmental VLANs with complete redundancy, dynamic routing, and enterprise security features.

The network infrastructure leverages industry-standard protocols including OSPF for dynamic routing, HSRP for gateway redundancy, Rapid PVST+ for loop prevention, and comprehensive security measures including SSH encryption, port security, DHCP snooping, and Dynamic ARP Inspection. The solution provides automatic failover capabilities at multiple levels ensuring high availability and business continuity.

## 1.2 Business Requirements

AfriTech Solutions required a robust, scalable network infrastructure to support their growing operations across two locations. The key business requirements included:

- Multi-site connectivity: Seamless communication between Nairobi HQ and Mombasa Branch

- Network segmentation: Separate networks for different departments (Management, DevOps, CyberSecurity, AI Research, Marketing, Remote VPN, Guest WiFi, Server Farm)

- High availability: Redundant core infrastructure and automatic failover

- Wireless access: Secure WiFi connectivity for mobile devices

- Security: Enterprise-grade security with access control and encryption

- Scalability: Ability to add additional sites and users

- Internet connectivity: Centralized internet access with ISP redundancy

## 1.3 Solution Summary

The implemented solution is a true WAN architecture featuring:

✓ Two geographically separated sites (Nairobi & Mombasa) ✓ 8 departmental VLANs with proper network segmentation ✓ OSPF dynamic routing with multi-area design ✓ HSRP gateway redundancy at each site ✓ Spanning Tree Protocol for loop prevention ✓ 8 wireless access points with WPA2 encryption ✓ SSH encryption on all infrastructure ✓ Port security, DHCP snooping, and DAI ✓ NAT/PAT for internet connectivity ✓ Dual ISP redundancy for internet access

This design provides a production-ready, enterprise-grade network that meets all business requirements while maintaining industry best practices for security, redundancy, and performance.

# 2. Network Architecture

## 2.1 Network Topology

The network follows a hierarchical three-tier architecture at each site (Core, Distribution, Access) with WAN connectivity between sites. The topology is designed for maximum redundancy and fault tolerance.

Internet Cloud ↓ ISP1 & ISP2 (Dual Redundancy) ↓ Edge Router (NAT + Firewall) ↙ ↓ ↘ Site1-Router ←WAN→ Site2-Router ↓ ↓ SITE 1 (Nairobi) SITE 2 (Mombasa) Core Switches (x2) Core Switches (x2) ↓ ↓ Dist Switches (x2) Dist Switches (x2) ↓ ↓ End Devices + APs End Devices + APs

## 2.2 Site Breakdown

### Site 1 - Nairobi Headquarters

Network: 172.16.0.0/16 (Class B)

VLANs:

| VLAN | Name | Subnet | Devices |
| --- | --- | --- | --- |
| 10 | Management | 172.16.10.0/24 | 25 |
| 20 | DevOps | 172.16.20.0/23 | 60 |
| 30 | CyberSecurity | 172.16.30.0/24 | 20 |
| 40 | AI Research | 172.16.40.0/23 | 70 |

### Site 2 - Mombasa Branch

Network: 192.168.0.0/16 (Class C ranges)

VLANs:

| VLAN | Name | Subnet | Devices |
| --- | --- | --- | --- |
| 50 | Marketing | 192.168.50.0/24 | 40 |
| 60 | Remote VPN | 192.168.60.0/24 | 20 |
| 100 | Guest WiFi | 192.168.100.0/24 | 50 |
| 200 | Server Farm | 192.168.200.0/24 | 35 |

## 2.3 Physical Infrastructure

Each site maintains identical redundant infrastructure:

- Core Layer: 2x Cisco 3560-24PS switches with Layer 3 routing, HSRP, and DHCP services

- Distribution Layer: 2x Cisco 2960-24TT switches with VLAN access ports and uplink redundancy

- Site Router: 1x Cisco 2911 for WAN connectivity and inter-site routing

- Wireless Infrastructure: 4x Access Points per site (8 total)

# 3. IP Addressing Scheme

The IP addressing scheme uses Class B addresses for Site 1 and Class C addresses for Site 2, with point-to-point /30 subnets for all WAN links.

## 3.1 Site 1 - Nairobi HQ (Class B: 172.16.0.0/16)

| VLAN | Department | Network | Gateway | Capacity |
| --- | --- | --- | --- | --- |
| VLAN 10 | Management | 172.16.10.0/24 | 172.16.10.1 | 254 hosts |
| VLAN 20 | DevOps | 172.16.20.0/23 | 172.16.20.1 | 510 hosts |
| VLAN 30 | CyberSecurity | 172.16.30.0/24 | 172.16.30.1 | 254 hosts |
| VLAN 40 | AI Research | 172.16.40.0/23 | 172.16.40.1 | 510 hosts |

## 3.2 Site 2 - Mombasa Branch (Class C: 192.168.0.0/16)

| VLAN | Department | Network | Gateway | Capacity |
| --- | --- | --- | --- | --- |
| VLAN 50 | Marketing | 192.168.50.0/24 | 192.168.50.1 | 254 hosts |
| VLAN 60 | Remote VPN | 192.168.60.0/24 | 192.168.60.1 | 254 hosts |
| VLAN 100 | Guest WiFi | 192.168.100.0/24 | 192.168.100.1 | 254 hosts |
| VLAN 200 | Server Farm | 192.168.200.0/24 | 192.168.200.1 | 254 hosts |

## 3.3 WAN Links and Router Interfaces

| Connection | Network |
| --- | --- |
| Site1-Router ↔ Core-SW1-Site1 | 10.1.1.0/30 |
| Site1-Router ↔ Core-SW2-Site1 | 10.1.1.4/30 |
| Site2-Router ↔ Core-SW1-Site2 | 10.1.2.0/30 |
| Site2-Router ↔ Core-SW2-Site2 | 10.1.2.4/30 |
| Site1-Router ↔ Site2-Router (WAN) | 10.0.0.0/30 |
| Site1-Router ↔ Edge-Router | 10.2.1.0/30 |
| Site2-Router ↔ Edge-Router | 10.2.2.0/30 |
| Edge-Router ↔ ISP1 | 203.0.113.0/30 |
| Edge-Router ↔ ISP2 | 198.51.100.0/30 |

Note: All WAN links use /30 subnets (4 IP addresses: network, 2 usable, broadcast) for optimal address utilization in point-to-point connections.

# 4. VLAN Design

## 4.1 VLAN Segmentation Strategy

The network uses VLANs to segment traffic based on departmental functions and security requirements. Each VLAN represents a separate broadcast domain with its own subnet and security policies.

Design Principles:

- Functional Separation: Each department has dedicated VLANs

- Security Zones: Sensitive departments (CyberSecurity, Server Farm) on isolated VLANs

- Guest Isolation: Guest WiFi completely isolated from internal networks

- Scalable Addressing: /23 subnets for high-user departments (DevOps, AI Research)

## 4.2 VLAN Distribution

| VLAN | Department | Distribution Switch | Users |
| --- | --- | --- | --- |
| 10 | Management | Dist-SW1-Site1 | 25 |
| 20 | DevOps | Dist-SW1-Site1 | 60 |
| 30 | CyberSecurity | Dist-SW2-Site1 | 20 |
| 40 | AI Research | Dist-SW2-Site1 | 70 |
| 50 | Marketing | Dist-SW1-Site2 | 40 |
| 60 | Remote VPN | Dist-SW1-Site2 | 20 |
| 100 | Guest WiFi | Dist-SW2-Site2 | 50 |
| 200 | Server Farm | Dist-SW2-Site2 | 35 |

# 5. Routing & Switching Protocols

## 5.1 OSPF Configuration

Open Shortest Path First (OSPF) provides dynamic routing between all sites with automatic convergence and load balancing. The network uses a multi-area OSPF design:

- Area 0 (Backbone): All WAN links, site routers, and edge router

- Area 1: Site 1 internal VLANs (172.16.0.0/16)

- Area 2: Site 2 internal VLANs (192.168.0.0/16)

OSPF Router IDs:

| Device | Router ID |
| --- | --- |
| Core-SW1-Site1 | 1.1.1.1 |
| Core-SW2-Site1 | 1.1.1.2 |
| Core-SW1-Site2 | 2.2.2.2 |
| Core-SW2-Site2 | 2.2.2.3 |
| Site1-Router | 10.1.1.1 |
| Site2-Router | 10.2.2.1 |
| Edge-Router | 10.10.10.10 |

Key Features:

- Passive interfaces on all VLAN interfaces (reduce overhead)

- Default route injection from Edge Router

- Automatic failover if primary paths fail

## 5.2 HSRP Gateway Redundancy

Hot Standby Router Protocol (HSRP) provides first-hop redundancy at each site. Core-SW1 serves as the active gateway with Core-SW2 as standby.

| VLAN | Virtual IP | Active | Standby |
| --- | --- | --- | --- |
| All Site 1 | xxx.xxx.xxx.1 | Core-SW1-Site1 (P:110) | Core-SW2-Site1 (P:100) |
| All Site 2 | xxx.xxx.xxx.1 | Core-SW1-Site2 (P:110) | Core-SW2-Site2 (P:100) |

## 5.3 Spanning Tree Protocol

Rapid PVST+ (Per-VLAN Spanning Tree Plus) prevents Layer 2 loops while maintaining per-VLAN load balancing:

- Root Bridge: Core-SW1 at each site (priority 4096)

- Secondary Root: Core-SW2 at each site (priority 8192)

- PortFast: Enabled on all access ports for immediate connectivity

- BPDU Guard: Protects against rogue switches on access ports

- Root Guard: Prevents topology changes from distribution switches

# 6. Network Services

## 6.1 DHCP Configuration

DHCP services are centralized on Core-SW1 switches at each site. Each VLAN has a dedicated DHCP pool with excluded addresses for infrastructure devices.

| VLAN | DHCP Pool Name | Range |
| --- | --- | --- |
| 10 | MANAGEMENT | 172.16.10.11 - 172.16.10.254 |
| 20 | DEVOPS | 172.16.20.11 - 172.16.21.254 |
| 30 | CYBERSECURITY | 172.16.30.11 - 172.16.30.254 |
| 40 | AI_RESEARCH | 172.16.40.11 - 172.16.41.254 |
| 50 | MARKETING | 192.168.50.11 - 192.168.50.254 |
| 60 | REMOTE_VPN | 192.168.60.11 - 192.168.60.254 |
| 100 | GUEST_WIFI | 192.168.100.11 - 192.168.100.254 |
| 200 | SERVER_FARM | 192.168.200.11 - 192.168.200.254 |

DHCP Configuration Details:

- Excluded Addresses: .1 to .10 reserved for infrastructure

- DNS Servers: 8.8.8.8 and 8.8.4.4 (Google Public DNS)

- Lease Time: Default 1 day

- IP Helper-Address: Configured on VLAN interfaces

## 6.2 DNS Services

All DHCP pools are configured to provide Google Public DNS servers (8.8.8.8 and 8.8.4.4) to end devices. This ensures reliable DNS resolution for internet connectivity without requiring internal DNS infrastructure.

## 6.3 NAT/PAT

Network Address Translation with Port Address Translation (NAT/PAT) is configured on the Edge Router to allow all internal devices to share the public IP addresses provided by the ISPs.

- Inside Networks: 172.16.0.0/16, 192.168.0.0/16, 10.0.0.0/8

- Outside Interface: GigabitEthernet 0/2 (primary ISP)

- NAT Type: PAT (overload) - thousands of internal IPs to one public IP

# 7. Wireless Infrastructure

## 7.1 Access Point Deployment

Eight wireless access points provide WiFi coverage across both sites, with department-specific SSIDs and VLAN assignments:

| AP Name | VLAN | SSID | Security | CH |
| --- | --- | --- | --- | --- |
| AP-Management-Site1 | 10 | AfriTech-Management-Site1 | WPA2-PSK/AES | 1 |
| AP-DevOps-Site1 | 20 | AfriTech-DevOps-Site1 | WPA2-PSK/AES | 6 |
| AP-CyberSec-Site1 | 30 | AfriTech-CyberSec-Site1 | WPA2-PSK/AES | 1 |
| AP-AI-Site1 | 40 | AfriTech-AI-Site1 | WPA2-PSK/AES | 6 |
| AP-Marketing-Site2 | 50 | AfriTech-Marketing-Site2 | WPA2-PSK/AES | 6 |
| AP-Remote-Site2 | 60 | AfriTech-Remote-Site2 | WPA2-PSK/AES | 11 |
| AP-Guest-Site2 | 100 | AfriTech-Guest-Site2 | WPA2-PSK/AES | 6 |
| AP-Servers-Site2 | 200 | AfriTech-Servers-Site2 | WPA2-PSK/AES | 11 |

## 7.2 Wireless Security

Security Configuration:

- Authentication: WPA2-PSK (strongest available in Packet Tracer)

- Encryption: AES (Advanced Encryption Standard)

- Password Policy: Department-specific 16+ character passwords

- Channel Planning: Non-overlapping channels (1, 6, 11)

- VLAN Assignment: Each AP assigned to appropriate VLAN

# 8. Security Implementation

## 8.1 Access Control

SSH Encryption:

- SSH version 2 with 2048-bit RSA keys on all infrastructure devices

- Telnet completely disabled

- Local user authentication with privilege 15

- Login banners warning unauthorized access

Access Control Lists (ACLs):

- CyberSecurity VLAN: No internet access (security policy)

- Guest WiFi: Isolated from internal networks

- Edge Router: Blocks unsolicited incoming traffic

## 8.2 Port Security

All access ports on distribution switches implement port security:

- MAC Address Sticky Learning: First devices automatically secured

- Maximum Devices: 2 per access port, 1 per server port

- Violation Action: Restrict (log and drop unauthorized traffic)

## 8.3 DHCP Snooping & Dynamic ARP Inspection

DHCP Snooping: Prevents rogue DHCP servers

- Enabled on all VLANs

- Uplink ports to core switches configured as trusted

- Access ports are untrusted by default

Dynamic ARP Inspection (DAI): Prevents ARP spoofing attacks

- Validates ARP packets against DHCP snooping binding table

- Uplink ports trusted for ARP

# 9. Equipment Specifications

## 9.1 Hardware Inventory

| Equipment | Model | Qty | Location |
| --- | --- | --- | --- |
| Core Switches | Cisco 3560-24PS | 4 | 2 per site |
| Distribution Switches | Cisco 2960-24TT | 4 | 2 per site |
| Site Routers | Cisco 2911 | 2 | 1 per site |
| Edge Router | Cisco 2911 | 1 | Central |
| ISP Routers | Cisco 2911 | 2 | ISP links |
| Access Points | AccessPoint-PT | 8 | 4 per site |
| PCs/Laptops | Generic | 41 | Both sites |
| Servers | Generic Server | 11 | Primarily Site 2 |
| Mobile Devices | Tablets/Phones | 48 | Both sites |

## 9.2 End Device Distribution

| VLAN | Department | Wired | Wireless | Servers | Total |
| --- | --- | --- | --- | --- | --- |
| 10 | Management | 6 | 6 | 1 | 13 |
| 20 | DevOps | 6 | 6 | 1 | 13 |
| 30 | CyberSec | 6 | 6 | 1 | 13 |
| 40 | AI Research | 6 | 6 | 1 | 13 |
| 50 | Marketing | 6 | 6 | 1 | 13 |
| 60 | Remote VPN | 6 | 6 | 1 | 13 |
| 100 | Guest WiFi | 5 | 6 | 1 | 12 |
| 200 | Server Farm | 0 | 6 | 4 | 10 |

Total End Devices: 100

# 10. Testing & Verification

## 10.1 Testing Methodology

A comprehensive testing plan validates all network functionality:

| # | Test | Expected Result |
| --- | --- | --- |
| 1 | Intra-VLAN connectivity (wired) | Successful ping |
| 2 | Intra-VLAN connectivity (wireless) | Successful ping |
| 3 | Wireless to wired same VLAN | Successful ping |
| 4 | Inter-VLAN same site | Successful ping via routing |
| 5 | Inter-site communication | Successful via WAN/OSPF |
| 6 | Guest network isolation | Ping fails (blocked) |
| 7 | CyberSec internet blocking | Internet ping fails |
| 8 | DHCP IP assignment | Device receives correct IP |
| 9 | Wireless DHCP | WiFi device gets IP |
| 10 | SSH access | Encrypted login successful |

## 10.2 Verification Results

OSPF Verification:

- All OSPF neighbors in FULL state

- Routing tables populated with all networks

- Inter-site routing functional via WAN link

HSRP Verification:

- Core-SW1 active on all VLANs

- Core-SW2 standby and ready for failover

- Virtual IP addresses responding to pings

DHCP Verification:

- All end devices receiving IP addresses

- Addresses in correct ranges for each VLAN

- Default gateways and DNS correctly assigned

# 11. Implementation Plan

## 11.1 Deployment Phases

Phase 1: Site 1 Core Infrastructure (Week 1)

- Install and configure core switches

- Configure VLANs, HSRP, and basic routing

- Test redundancy and failover

Phase 2: Site 1 Distribution & Access (Week 2)

- Deploy distribution switches

- Configure port security and DHCP snooping

- Deploy wireless access points

- Migrate end devices

Phase 3: Site 2 Infrastructure (Week 3)

- Replicate Site 1 configuration at Site 2

- Deploy all switches and access points

- Test local connectivity

Phase 4: WAN Connectivity (Week 4)

- Configure site routers

- Establish WAN link

- Configure OSPF routing

- Test inter-site connectivity

Phase 5: Internet & Security (Week 5)

- Configure Edge Router

- Establish ISP connections

- Configure NAT and firewall ACLs

- Enable SSH on all devices

## 11.2 Risk Mitigation

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Configuration errors | High | Peer review all configs before deployment |
| Network downtime | High | Deploy during off-hours, maintain fallback |
| Security vulnerabilities | Medium | Regular security audits, patch management |
| ISP failure | Medium | Dual ISP with automatic failover |
| Hardware failure | Medium | Redundant infrastructure at all levels |

# 12. Conclusion & Recommendations

## 12.1 Project Summary

The AfriTech Solutions enterprise WAN network successfully meets all business requirements with a robust, scalable, and secure multi-site architecture. The implementation provides:

- High Availability: Redundant infrastructure at all levels

- Automatic Failover: HSRP, OSPF, and dual ISP redundancy

- Security: Multi-layered security with SSH, ACLs, port security

- Scalability: Easily expandable to additional sites

- Performance: Dynamic routing and load balancing

The network supports 320 end devices across 8 VLANs with complete separation of departmental traffic. All devices can communicate as needed while maintaining security boundaries between departments.

## 12.2 Future Enhancements

Short-term Recommendations (3-6 months):

- Implement centralized logging with Syslog server

- Deploy SNMP for network monitoring and management

- Configure NTP for accurate time synchronization

- Implement QoS for VoIP and critical applications

Medium-term Recommendations (6-12 months):

- Upgrade to RADIUS/TACACS+ for AAA authentication

- Implement 802.1X for network access control

- Deploy wireless controller for centralized AP management

- Add IPS/IDS for threat detection

Long-term Recommendations (12+ months):

- Consider SD-WAN for additional sites

- Implement BGP for ISP connections

- Add disaster recovery site

- Upgrade to next-generation firewalls